KL4030 Based 3&4Axis CNC Packages

Simplified Wiring Diagram

Parallel Port IO

This can be very sysem specific,but for a common 3-axis parall port controlled CNC device,the following chart can be a starting point.

| Pn | Direction | Purpose |
| --- | --- | --- |
| 1 | Output | Spindle Control Relay |
| 2 | Output | X Step |
| 3 | Output | X Direction |
| 4 | Output | Y Step |
| 5 | Output | Y Direction |
| 6 | Output | Z Step |
| 7 | Output | Z Direction |
| 10 | Input | E-Stop |
| 11 | Input | Limit X |
| 12 | Input | Limit Y |
| 13 | Input | Limit Z |

Wiring

JP1 and Jp2 are used to selects the 5v  power supply or 10v power supply.When you uesd 5v power supply,you must Plug JP1 and JP2.else you must pull out the JP1 and JP2.

EU AC Wire Colors

Ground, or Earth is always Green/Yellow

Live is Always Brown

Neutral is Blue

US AC Wire Colors

Ground, or Earth is Usually Green

Live is usuall Black

Neutral is usually White